Information regarding Fuel injection pre- filtering- Fiat FI systems.

It is recommended to install a pre-filter before the fuel pump on a Bosch fuel-injected Fiat. The stock system is set up with the fuel filter after the fuel pump, and grit or sediment can lock up the mini-bearings in the pump.

When installing a Pre-pump filter can use WIX 33299, crosses to Napa 3299- has large orifices on both ends to accommodate fitment to large fuel inlet hose. Also can use Alfa filter pn  604 14984 Z

From Richard Ridge  who uses:

The part is Fram G6399.  NAPA lists it as  FIL3278. The original application was an '84 Audi 4000, where it is used as a prefilter.

Fuel Pump options:

Autozone lists pn E2182 (universal New replacement)- list as of  10-25-05 $110.99

New Bosch Fuel pump, 124, old # 0580 464 008, supercedes to 61408, list as of

6-05, $414.00- Advise Searching various vendors for best pricing

X19- Bosch 61413   Can swap back and forth

Reman option, Royze  # 15E30056 list as of 6-05  $146

Main Fuel Filter pn 71028