Fiat Fan Temperature switches- Options

OE Fiat part number- 4318331

With molded wire ends-  4424393   (Either will work with butt connector modification.

Switch on @  92 C / 197.6 F      Switch off @ 87 C  /  188.6 F

Aftermarket/ OE - The numbers are OE Wahler but the same in OE Volkswagen also.

Switch on                                        Switch off

823 959 481 F         92 C / 1976F                                    87C/ 188.6F

823 959 481 D        82 C /  179.6F                                  77C/ 170.6 F

( this is the one preferred by many as a replacement for the stock fan switch)

823 959 481            75C / 167F                                       70C / 158   F

( probably ok for the tropics )

**note 81-84 Saab turbo uses the same switch  – SAAB part number  8547572/82  but this number will generally supercede to the above listed 823 859 481 D in most European databases.